SECTION 7

## Wireless Network Types

Wireless networking removes the need for physical cabling and provides flexibility, scalability, and faster deployment than wired networks. The wireless network type you choose affects performance, reliability, coverage, and management, so understanding the differences is essential for network technicians and administrators.

The four primary wireless network types are:

- Ad hoc

- Infrastructure

- Point-to-point

- Mesh

This lesson also explains autonomous vs lightweight access points.

## 1) Ad Hoc Wireless Networks (IBSS)

An ad hoc wireless network, also called an Independent Basic Service Set (IBSS), allows devices to connect directly to each other without using a centralized wireless access point.

This configuration operates in peer-to-peer mode, meaning each device communicates directly with the others rather than through network infrastructure. Because no access point is required, ad hoc networks are quick to deploy and useful for temporary or short-term connections.

Ad hoc networks typically do not provide internet access, since they are isolated from other networks unless one device shares its connection. All devices must be within wireless range of one another.

Use cases

- Quickly sharing files between two devices

- Temporary collaboration

- Situations where no wireless infrastructure exists

Key limitations

- Limited range

- Limited scalability

- No centralized management

- Usually no internet connectivity

## 2) Infrastructure Wireless Networks

An infrastructure wireless network uses one or more wireless access points (APs) to connect wireless devices to a wired local area network (LAN).

In this setup, laptops, phones, and other wireless clients connect to the access point instead of directly to each other. The access point then bridges wireless traffic onto the wired network.

Each access point has a BSSID (Basic Service Set Identifier), which is typically the MAC address of the access point. This uniquely identifies each AP at the wireless level.

In addition to the BSSID, the network uses an SSID (Service Set Identifier), which is the human-readable name of the wireless network that users see when searching for Wi-Fi.

### Single Access Point (BSS)

Most home networks use a single access point, forming one Basic Service Set (BSS).

### Multiple Access Points (ESS)

In larger homes or office buildings, multiple access points are deployed for coverage. These access points share the same SSID, forming an Extended Service Set (ESS).

When using ESS:

- All APs share the same SSID

- Clients roam seamlessly between access points

- The SSID becomes an ESSID (Extended Service Set Identifier)

Use cases

- Homes

- Offices

- Schools

- Enterprises

Advantages

- Centralized access

- Internet connectivity

- Scalable

- Easier management than ad hoc networks

## 3) Point-to-Point Wireless Networks

A point-to-point wireless network connects two fixed locations using directional, high-gain antennas, often over long distances.

Unlike ad hoc networks, point-to-point links are static, meaning both ends are permanently installed and aligned. These links provide dedicated bandwidth between the two locations.

Point-to-point wireless is commonly used when running physical cabling is impractical or impossible.

### Example scenario

A microwave wireless link connects a home or building to a distant central location that has fiber internet access. One fiber connection serves the central site, and wireless point-to-point links distribute connectivity outward.

Use cases

- Connecting two buildings

- Long-distance wireless bridges

- Rural or hard-to-reach locations

Advantages

- High efficiency

- Dedicated bandwidth

- Long-range connectivity

## 4) Mesh Wireless Networks

A wireless mesh network consists of multiple interconnected nodes where each node can connect to multiple other nodes. Data can travel through multiple possible paths, making the network self-forming and self-healing.

If one path fails, traffic automatically reroutes through another path. This makes mesh networks extremely resilient and reliable.

### Types of mesh networks

#### a) Infrastructure-based mesh

- Multiple access points operating together

- Often uses ESS configuration

- Provides full coverage across large buildings

- Clients roam seamlessly

#### b) Hybrid / multi-technology mesh

Combines multiple wireless technologies, such as:

- Wi-Fi

- Cellular

- Microwave links

- Satellite connections

### Disaster-relief example

In disaster zones where normal infrastructure is destroyed:

- Satellite provides primary internet access

- Wired networking connects core systems

- Wi-Fi provides local access

- Cellular towers restore mobile service

- Point-to-point microwave links connect remote outposts

All of these form a single mesh network with redundancy. If one link fails, traffic reroutes automatically.

Advantages

- Self-healing

- Highly resilient

- Ideal for large or temporary deployments

- No need for extensive cabling

## Autonomous vs Lightweight Access Points

### Autonomous Access Points

An autonomous access point is a standalone device that handles all wireless functions independently.

- Configured individually

- No central controller required

- Common in home and small office environments

Pros

- Simple

- No controller needed

Cons

- Harder to manage at scale

- Configuration must be repeated on each AP

### Lightweight Access Points

A lightweight access point relies on a centralized wireless controller.

- The controller manages configuration, security, and policies

- APs offload processing to the controller

- Common in enterprise networks

Pros

- Centralized management

- Easier updates and policy enforcement

- Scales well for large networks

Cons

- Requires a controller

- Higher initial complexity

## Final Key Takeaway

Choosing the correct wireless network type depends on:

- Size of the environment

- Required reliability

- Coverage needs

- Ease of management

- Available infrastructure

Each type has a specific purpose:

- Ad hoc → quick, temporary connections

- Infrastructure → standard Wi-Fi networks

- Point-to-point → long-distance fixed links

- Mesh → resilient, large-scale, flexible network

## Wireless Antennas

Wireless antennas are a critical component of wireless communication systems because they control how far and in which direction radio frequency (RF) signals are sent and received.

Antennas do not create signal — they shape and direct it.
The type of antenna you choose directly affects:

- Coverage area

- Signal strength

- Range

- Efficiency

- User experience

The three antenna types covered in this lesson are:

- Omnidirectional antennas

- Unidirectional (directional) antennas

- Yagi antennas

## 1️⃣ Omnidirectional Antennas

### What they are

An omnidirectional antenna transmits and receives wireless signals equally in all directions (360 degrees on a horizontal plane).

### How they behave

- Signal spreads outward evenly

- No focus in a single direction

- Shorter range compared to directional antennas

- Designed for broad area coverage

### Where they are commonly used

- Wireless access points

- Laptops and phones

- Mobile hotspots

- Public Wi-Fi (coffee shops, hotels, convention centers)

- Home and office Wi-Fi networks

### When to use them

Use omnidirectional antennas when:

- You want to serve many clients around a central location

- Clients are spread in all directions

- You want consistent coverage rather than long distance

### Key takeaway

Omnidirectional antennas are best for centralized Wi-Fi coverage where users are located in all directions.

## 2️⃣ Unidirectional (Directional) Antennas

### What they are

A unidirectional antenna focuses most of its energy in a single direction, creating a stronger and more concentrated signal beam.

### How they behave

- Narrower coverage area

- Much longer range than omnidirectional antennas

- Less signal wasted in unwanted directions

- Higher signal strength in the target direction

### Where they are commonly used

- Point-to-point wireless links

- Building-to-building connections

- Long corridors or narrow coverage zones

- Extending internet to remote structures

### Real-world example (house → barn)

- House has internet and network equipment

- Barn is half a mile away, no cabling

- Digging a trench for fiber is impractical

- Two unidirectional antennas are installed:

  - One on the house, pointed at the barn

  - One on the barn, pointed at the house

- This creates a wireless point-to-point link

- An access point in the barn then provides Wi-Fi locally

### When to use them

Use unidirectional antennas when:

- You need to reach a specific location

- You want long-distance communication

- You are creating a point-to-point network

### Key takeaway

Unidirectional antennas are ideal for long, narrow coverage areas or fixed point-to-point links.

## 3️⃣ Yagi Antennas

### What they are

A Yagi antenna is a specific type of unidirectional antenna designed to provide:

- Very high signal gain

- Very narrow beam width

- Excellent long-distance performance

### How they behave

- Highly directional

- Extremely focused signal

- Minimal interference from other directions

- Must be carefully aimed

### Physical design

A Yagi antenna consists of:

- Multiple parallel metal elements

- Arranged in a straight line

- Designed to amplify signals from one direction

### Where they are commonly used

- Connecting to distant ISP or cell towers

- Rural or remote internet access

- Television signal reception

- Long-distance building connections

### When to use them

Choose a Yagi antenna when:

- Distance is very large

- Target location is fixed

- Maximum signal strength is required

- Precision matters more than coverage area

### Relationship to unidirectional antennas

- All Yagi antennas are unidirectional

- Not all unidirectional antennas are Yagi

### Key takeaway

A Yagi antenna is a high-gain, narrow-beam directional antenna used for long-distance, fixed connections.

## Choosing the Right Antenna (Big Picture)

Your antenna choice depends on:

- Coverage area needed

- Distance to clients or targets

- Directionality requirements

- Physical environment

- Network design goals

### Summary Comparison

- Omnidirectional

  - Wide coverage

  - Shorter range

  - Many users around one point

- Unidirectional

  - Focused coverage

  - Longer range

  - Point-to-point or narrow areas

- Yagi

  - Extremely focused

  - Very long range

  - Fixed, distant targets

## Final Exam-Ready Takeaway (Network+)

Wireless antennas shape RF signals. Omnidirectional antennas provide broad coverage, unidirectional antennas focus signals in one direction, and Yagi antennas provide high-gain, narrow-beam communication for long distances.

## Wireless Antennas

Wireless antennas control how radio signals travel:

- Which direction

- How far

- How concentrated

They don’t create signal — they shape and focus RF energy.
Different antennas exist because different situations need different signal behavior.

## 1️⃣ Omnidirectional Antennas (Most Common)

### What “omnidirectional” means

- Omni = all

- Sends and receives signal equally in all directions

- 360° coverage on the horizontal plane

### Why phones use omnidirectional antennas

A cell phone:

- Doesn’t know where the signal will come from

- Needs to connect while moving

- Must work whether the tower is in front, behind, left, or right

So it sends and receives RF in every direction with equal power.

This is true for:

- Phones

- Laptops

- Tablets

- Wireless access points

- Hotspots

### Wireless access points (APs)

Most home APs:

- Have internal omnidirectional antennas

- No visible external antenna

- Signal radiates outward in all directions

📌 Placement matters
If placed near a wall:

- Signal leaks into neighboring apartments

- Potential security and interference issue

Best practice:

- Place AP near the center of the space

### External omnidirectional antennas

Some devices allow antenna replacement:

- Small “whip” antennas = omnidirectional

- Bigger antenna = more range, same 360° pattern

🔑 Longer antenna ≠ directional
It just means more propagation, not focus.

### Wireless cards & penetration testing

External Wi-Fi adapters:

- Allow antenna swapping

- Omnidirectional antenna → broad scanning

- Used when you want coverage in all directions

### When to use omnidirectional

Use when:

- Clients are all around you

- You want broad area coverage

- Mobility is important

## 2️⃣ Unidirectional (Directional) Antennas

### What “unidirectional” means

- Uni = one

- Focuses signal in a single direction

- Stronger signal

- Longer distance

- Little to no signal behind the antenna

### Why direction matters

Omnidirectional antennas:

- Waste power in all directions

Directional antennas:

- Concentrate power

- Go much farther

- Reduce interference

This is why directional antennas are preferred for:

- Long distances

- Fixed targets

- Point-to-point links

### Real-world example (house → barn)

- House has internet

- Barn is ½ mile away

- No fiber or copper installed

- Trenching is expensive

Solution:

- One directional antenna on the house

- One directional antenna on the barn

- Pointed directly at each other

Result:

- Point-to-point wireless link

- Internet extended to the barn

- Local AP inside barn provides Wi-Fi

### When to use unidirectional

Use when:

- Target location is fixed

- Distance is long

- Coverage area is narrow

- You want to avoid signal bleed

Examples:

- Building-to-building

- Long corridors

- Remote structures

## 3️⃣ Parabolic Antennas (Special Directional)

### What parabolic means

- A curved dish

- Focuses RF energy to a precise point

- Very high gain

- Very narrow beam

Commonly used for:

- Satellite TV

- Microwave links

- Long-distance backhaul

The curve:

- Collects scattered signals

- Focuses them toward one direction (like a mirror)

📌 Think: satellite dish

## 4️⃣ Yagi Antennas (Very Important for Exam)

### What a Yagi antenna is

A Yagi antenna is:

- A type of unidirectional antenna

- Very narrow beam

- Very high gain

- Long distance

Physical design:

- Looks like an arrow

- Multiple metal elements in a straight line

- Must be carefully aimed

### Where Yagi antennas are used

- TV signal reception (UHF/VHF)

- Long-distance Wi-Fi links

- Campus building connections

- Business parks

📌 Still used today for:

- Wireless point-to-point links

- Fixed, long-distance targets

### Key relationship

- All Yagi antennas are directional

- Not all directional antennas are Yagi

## 5️⃣ Exam-Relevant Antenna Types to Remember

### Omnidirectional

- Phones

- APs

- Hotspots

- Broad coverage

- Shorter range

### Directional (Unidirectional)

- Point-to-point

- Building-to-building

- Focused beam

- Longer range

### Yagi

- High gain

- Narrow beam

- Fixed direction

- Long distance

### Parabolic

- Dish-shaped

- Satellite & microwave

- Extremely focused

## Final Exam Takeaway (Memorize This)

Omnidirectional antennas provide 360° coverage for local users, while directional antennas (including Yagi and parabolic) focus RF energy in one direction for long-distance, fixed wireless links.

Wireless Frequencies (Wi-Fi Bands)

What “Wireless Frequencies” Means

Wireless frequencies refer to the frequency bands used to transmit and receive radio waves in wireless networks.
Each wireless transmission must operate at a specific frequency, and each frequency band has unique characteristics that affect:

- Speed

- Range

- Penetration through objects

- Interference

- Number of usable channels

These frequency bands are regulated by governments to prevent interference between services such as Wi-Fi, radar, medical equipment, and military systems.

The Three Wi-Fi Frequency Bands

Wi-Fi uses three main frequency bands:

- 2.4 GHz

- 5 GHz

- 6 GHz

Each has trade-offs between speed and distance.

1️⃣ 2.4 GHz Frequency Band

Overview

- In use since ~1997

- One of the most widely used Wi-Fi bands

- Known for long range and good penetration

Frequency Range

- Approximately 2.400 GHz – 2.495 GHz

- Commonly referred to simply as 2.4 GHz

Channels in 2.4 GHz

- Bandwidth allocated by government: 72 MHz

- Channel width: 22 MHz

- Channels overlap heavily

Depending on region:

- US / Canada → Channels 1–11

- Most of world → Channels 1–13

- Japan → Channels 1–14

📌 Only 3 non-overlapping channels:

- Channel 1

- Channel 6

- Channel 11

👉 This is exam-critical and must be memorized

Why Overlap Happens

- 72 MHz total bandwidth

- Each channel uses 22 MHz

- Too many channels share too little space

- Overlap causes interference and retransmissions

Strengths of 2.4 GHz

- Longest range

- Best wall penetration

- Works well through drywall, studs, obstacles

Weaknesses of 2.4 GHz

- Slower speeds

- Heavy interference

- Few non-overlapping channels

2️⃣ 5 GHz Frequency Band

Why 5 GHz Was Introduced

Due to congestion and overlap in 2.4 GHz, newer Wi-Fi standards began using 5 GHz, which has:

- More spectrum

- Less interference

- Better performance

Frequency Range

- Approximately 5.725 GHz – 5.875 GHz

- Referred to as 5 GHz

Channels in 5 GHz

- Much larger spectrum allocated

- Up to 24 non-overlapping channels

- Default channel width: 20 MHz

This allows:

- Higher performance

- Less interference

- Better support for dense environments

Channel Bonding (Very Important)

5 GHz supports channel bonding, where multiple channels are combined:

- 20 MHz (default)

- 40 MHz (2 bonded)

- 80 MHz (4 bonded)

- 160 MHz (8 bonded)

✅ Benefits:

- Higher throughput

- Faster speeds (video, gaming, streaming)

❌ Downsides:

- Fewer non-overlapping channels

- Increased susceptibility to interference

Example:

- 24 channels → 12 → 6 → 3 as bonding increases

Strengths of 5 GHz

- Faster speeds than 2.4 GHz

- Less congestion

- More channels

- Ideal for high-bandwidth applications

Weaknesses of 5 GHz

- Shorter range

- Poorer wall penetration than 2.4 GHz

3️⃣ 6 GHz Frequency Band (Wi-Fi 6E)

Overview

- Newest Wi-Fi band

- Designed for high-density, high-performance networks

- Very low congestion

Frequency Range

- Approximately 5.925 GHz – 7.125 GHz

- Referred to as 6 GHz

Spectrum Allocation

- 1200 MHz allocated by governments

- Largest spectrum of all Wi-Fi bands

Channels in 6 GHz

Channel widths supported:

- 20 MHz

- 40 MHz

- 80 MHz

- 160 MHz

Number of channels:

- Up to 59 channels at 20 MHz

- As few as 7 channels at 160 MHz

Strengths of 6 GHz

- Fastest Wi-Fi speeds

- Lowest interference

- Ideal for dense environments

- Excellent for latency-sensitive apps

Weaknesses of 6 GHz

- Shortest range

- Least wall penetration

- Requires newer devices

Government Regulation & 802.11h

Why Regulation Matters

Governments allocate wireless spectrum to avoid interference between:

- Wi-Fi

- Radar

- Medical equipment

- Military systems

802.11h (Important Concept)

Introduced due to interference issues with radar systems, especially in Europe.

Key features:

🔹 Dynamic Frequency Selection (DFS)

- Devices monitor for radar signals

- If detected → automatically switch channels

- Prevents interference with radar

🔹 Transmit Power Control (TPC)

- Adjusts power to minimum required level

- Reduces interference

- Saves battery life

These features are now widely adopted globally.

Band Steering

What Band Steering Is

Band steering is a wireless management feature that:

- Automatically moves capable devices to faster, less congested bands

- Typically from 2.4 GHz → 5 GHz or 6 GHz

How It Works

- Access point detects client capabilities

- Pushes dual-band or tri-band clients to faster bands

- Leaves 2.4 GHz for legacy devices

Benefits

- Better performance

- Reduced congestion

- Efficient spectrum usage

- Improved user experience

Final Comparison Summary (Exam Focus)

2.4 GHz

- Slowest speeds

- Longest range

- Best penetration

- Most interference

5 GHz

- Faster than 2.4 GHz

- Shorter range

- Channel bonding supported

- Less interference

6 GHz

- Fastest speeds

- Shortest range

- Most spectrum

- Least interference

Final Exam-Ready Takeaway

Wi-Fi operates in the 2.4 GHz, 5 GHz, and 6 GHz bands, each offering different trade-offs between speed, range, channel availability, and interference, all regulated by government spectrum allocation.

## Note: 

1️⃣ Start With the Frequency Band

Take the 2.4 GHz Wi-Fi band:

- Band range ≈ 2.400 GHz to 2.495 GHz

- Total usable spectrum ≈ 72 MHz

This entire range is too wide for one transmission, so it’s divided into channels.

3️⃣ Example: Channel 1 in the 2.4 GHz Band

- Center frequency: 2.412 GHz

- Channel width: ~22 MHz

That means Channel 1 actually spans:

- From about 2.401 GHz

- To about 2.423 GHz

So even though we name it “2.412 GHz,” it really uses a range.

4️⃣ Why Channels Overlap

Channels are spaced 5 MHz apart, but each channel is 22 MHz wide.

Example:

- Channel 1 center: 2.412 GHz

- Channel 2 center: 2.417 GHz

Since 22 MHz > 5 MHz:
👉 They overlap

That’s why interference happens.

Key Idea (This Is the Missing Link 🔑)

Wi-Fi does NOT send data on just one single frequency inside the channel.

Instead:

Wi-Fi splits a channel into MANY tiny frequencies and sends data on all of them at the same time.

This is the reason wider channels = more data.

About 5GHz band 

1️⃣ The 5 GHz band is NOT one continuous block

The 5 GHz Wi-Fi band is split into multiple smaller sub-bands (UNII bands), each with its own limits.

Examples (simplified):

- UNII-1

- UNII-2

- UNII-2e

- UNII-3

Each sub-band is typically 80–160 MHz wide (some smaller).

Construction of 5 GHz UNII Sub-Bands

Below is a clean, exam-safe breakdown.

1️⃣ UNII-1 (Lower Band)

Frequency range:

- 5.150 – 5.250 GHz

- Width ≈ 100 MHz

Characteristics:

- Indoor use

- Lower transmit power

- No DFS required

Channels:

- 20 MHz channels: 36, 40, 44, 48

Why it exists:

- Safe indoor Wi-Fi

- Minimal interference with radar

2️⃣ UNII-2 (Middle Band)

Frequency range:

- 5.250 – 5.350 GHz

- Width ≈ 100 MHz

Characteristics:

- Indoor & outdoor

- DFS required

- Moderate power

Channels:

- 52, 56, 60, 64

Why it exists:

- Adds more spectrum

- Shared with weather & military radar → DFS protection

3️⃣ UNII-2e (Extended UNII-2)

Frequency range:

- 5.470 – 5.725 GHz

- Width ≈ 255 MHz

Characteristics:

- Indoor & outdoor

- DFS required

- Large contiguous space

Channels:

- 100 → 144

Why it exists:

- Provides most of the usable 5 GHz capacity

- Enables 80 MHz and some 160 MHz channels

⚠️ DFS makes it unstable in radar areas

4️⃣ UNII-3 (Upper Band)

Frequency range:

- 5.725 – 5.875 GHz

- Width ≈ 150 MHz

Characteristics:

- Outdoor-friendly

- Higher transmit power

- No DFS required

Channels:

- 149, 153, 157, 161, 165

Why it exists:

- Long-range outdoor links

- Point-to-point wireless bridges

- Cleaner than DFS bands

❌ Too small for a full contiguous 160 MHz channel

Putting It All Together (Mental Map)

5.150 ── UNII-1 ── 5.250 ── UNII-2 ── 5.350

|

UNII-2e (wide)

|

5.725 ── UNII-3 ── 5.875

Why Sub-Bands Matter (Big Picture)

| Sub-band | DFS | Power | Best use |
| --- | --- | --- | --- |
| UNII-1 | ❌ | Low | Indoor Wi-Fi |
| UNII-2 | ✅ | Medium | Extra channels |
| UNII-2e | ✅ | Medium | High capacity |
| UNII-3 | ❌ | High | Outdoor / bridges |

Exam-Ready One-Sentence Summary

The 5 GHz Wi-Fi band is divided into UNII-1, UNII-2, UNII-2e, and UNII-3 sub-bands, each with different frequency ranges, power limits, and DFS requirements to manage interference and spectrum sharing.

Wireless Standards (IEEE 802.11)

Overview

Wireless networking standards are defined by the IEEE (Institute of Electrical and Electronics Engineers) under the 802.11 family of standards. These standards specify how wireless local area networks (WLANs) such as Wi-Fi operate.

Over time, newer 802.11 standards were introduced to:

- Increase data transfer speeds

- Improve reliability

- Support more devices

For the exam, you must know for each standard:

- Name (a, b, g, n, ac, ax)

- Frequency band(s) used

- Maximum speed

- Approximate range

1️⃣ Wireless a (802.11a)

- Frequency band: 5 GHz

- Maximum speed: 54 Mbps

- Range: ~35 meters

Key points:

- Introduced in the late 1990s

- Required expensive 5 GHz radios

- Primarily adopted by businesses

- Limited home adoption due to cost

2️⃣ Wireless b (802.11b)

- Frequency band: 2.4 GHz

- Maximum speed: 11 Mbps

- Range: ~140 meters

Why it mattered:

- 2.4 GHz radios were cheap and widely available

- Shared spectrum with household devices (microwaves, baby monitors, walkie-talkies, cameras)

- Enabled mass adoption of Wi-Fi in homes, schools, and businesses

- Much slower than wireless a, but considered fast at the time

3️⃣ Wireless g (802.11g)

- Frequency band: 2.4 GHz

- Maximum speed: 54 Mbps

- Range: ~140 meters

Key advantage:

- Same speed as wireless a

- Same low-cost 2.4 GHz band as wireless b

- Became the dominant standard for many years

4️⃣ Wireless n (802.11n / Wi-Fi 4)

- Frequency bands:

  - 5 GHz (primary for high speed)

  - 2.4 GHz (for backward compatibility)

- Maximum speed: Up to 600 Mbps

- Range:

  - ~35 meters (5 GHz)

  - ~70 meters (2.4 GHz)

Backward Compatibility Issue

- Early 5 GHz-only deployments were incompatible with b/g devices

- Solution: Dual-radio access points

  - 2.4 GHz radio → supports b/g/n

  - 5 GHz radio → supports faster n

New Technology Introduced: MIMO

- MIMO (Multiple-Input Multiple-Output)

- Uses multiple antennas to transmit data in parallel

- More antennas = more simultaneous data streams

- Data is split, transmitted, then recombined

5️⃣ Wireless ac (802.11ac / Wi-Fi 5)

- Frequency band: 5 GHz only

- Maximum speed: Up to 6.9 Gbps

Important Exam Truth

- 802.11ac officially supports ONLY 5 GHz

- No 2.4 GHz support in the standard

New Technology: MU-MIMO

- MU-MIMO (Multi-User MIMO)

- Allows multiple devices to communicate simultaneously

- MIMO = one user at a time (shared)

- MU-MIMO = behaves like a switch, not a hub

- Reduces collisions and congestion

6️⃣ Wireless ax (802.11ax / Wi-Fi 6 & Wi-Fi 6E)

- Frequency bands:

  - 2.4 GHz

  - 5 GHz

  - 6 GHz (Wi-Fi 6E)

- Maximum speed: Up to 9.6 Gbps

Key characteristics:

- Introduced around 2021

- Uses advanced MU-MIMO

- Access points contain three radios

- Fully backward compatible with a, b, g, n, ac

Frequency Band Support Summary (EXAM CRITICAL)

2.4 GHz supported by:

- b

- g

- n

- ax

5 GHz supported by:

- a

- n

- ac

- ax

6 GHz supported by:

- ax (Wi-Fi 6E only)

Dual-Band Standards (Official)

According to the IEEE standard, only these are dual-band:

- Wireless n (2.4 + 5 GHz)

- Wireless ax (2.4 + 5 + 6 GHz)

All others are single-band only.

Distance vs Speed Rule

As standards evolve:

- Speed increases

- Coverage distance decreases

This is due to higher frequencies and wider channels.

Exam-Style Scenario Examples

Example 1

Question: Which standards do NOT support 5 GHz?

- a, b, g, n

Answer:

- b and g

Example 2 (Troubleshooting)

An old laptop with a wireless b card cannot connect to a wireless ac network.

Cause:

- Frequency mismatch

- Wireless ac = 5 GHz only

- Wireless b = 2.4 GHz only

Marketing vs Reality (VERY IMPORTANT)

What stores say:

- “Wireless ac supports 2.4 GHz and 5 GHz”

What the standard says:

- False

The truth:

- The access point has:

  - One 5 GHz radio running 802.11ac

  - One 2.4 GHz radio running 802.11n

The 2.4 GHz support comes from 802.11n, not 802.11ac.

Final Exam Rule 🔒

Always answer based on IEEE standards, not marketing labels.

Final One-Line Lock-In

Only wireless n and wireless ax are officially dual-band standards under IEEE 802.11.

NOTE:

MIMO vs MU-MIMO (Focused Comparison)

| Aspect | MIMO | MU-MIMO |
| --- | --- | --- |
| Who gets data at once | One user | Multiple users |
| Antennas used | All for one device | Shared across devices |
| Transmission | Parallel streams to one client | Parallel streams to many clients |
| Main benefit | Speed for one device | Efficiency for many devices |

Exam-Ready One-Sentence Answer 🔒

MU-MIMO works by using multiple antennas and beamforming to transmit separate spatial streams to different users simultaneously on the same channel.

| Older → Newer | Speed | Distance |
| --- | --- | --- |
| b → g → n → ac → ax | ⬆ Increases | ⬇ Decreases |

| Standard Name | IEEE Name | Wi-Fi Gen | Frequency Band(s) | Max Speed | Approx. Range | Key Notes (Exam-Critical) |
| --- | --- | --- | --- | --- | --- | --- |
| Wireless a | 802.11a | — | 5 GHz | 54 Mbps | ~35 m | Expensive radios, business use, 5 GHz only |
| Wireless b | 802.11b | — | 2.4 GHz | 11 Mbps | ~140 m | Cheap radios, household devices share band |
| Wireless g | 802.11g | — | 2.4 GHz | 54 Mbps | ~140 m | Same speed as a, cheaper band |
| Wireless n | 802.11n | Wi-Fi 4 | 2.4 GHz & 5 GHz | Up to 600 Mbps | ~70 m (2.4) / ~35 m (5) | Dual-band, introduced MIMO |
| Wireless ac | 802.11ac | Wi-Fi 5 | 5 GHz only | Up to 6.9 Gbps | Shorter than n | MU-MIMO, NOT dual-band by standard |
| Wireless ax | 802.11ax | Wi-Fi 6 / 6E | 2.4, 5, 6 GHz | Up to 9.6 Gbps | Shortest range | MU-MIMO, backward compatible |

# 🔐 Wireless Security

## 1️⃣ Why Wireless Security Matters

Wireless networks are inherently less secure than wired networks because:

- Signals travel through the air

- Anyone within the signal footprint can attempt to connect

- Attackers can use phones, tablets, or laptops

To protect wireless networks, two things are required:

- Authentication – who is allowed to connect

- Encryption – how data is protected while transmitted

## 2️⃣ Wireless Authentication Methods

### A) Pre-Shared Key (PSK)

- Same password/key used by:

  - Wireless Access Point

  - All client devices

- Configured in advance

- Used to establish an encrypted connection

Where it’s commonly used

- Home networks

- Small offices

Problems with PSK

- ❌ Poor scalability (many users share one password)

- ❌ Secure distribution is difficult

- ❌ If one user leaves, the key must be changed for everyone

- ❌ No individual user accountability

- ❌ Cannot track who did what on the network

📌 Exam association

- PSK = Personal mode

- Think: single shared password

### B) Enterprise Authentication (802.1X)

- Uses individual credentials

  - Username & password or

  - Digital certificates

- Relies on a centralized authentication server

Authentication server

- RADIUS (Remote Authentication Dial-In User Service)

## 🔐 RADIUS = Authentication Server

RADIUS (Remote Authentication Dial-In User Service) is a centralized authentication server used in enterprise networks.

### What RADIUS does

A RADIUS server:

- ✅ Authenticates users (verifies username/password or certificate)

- ✅ Authorizes access (what the user is allowed to do)

- ✅ Accounts for activity (logs who connected, when, and how long)

This is often summarized as AAA:

- Authentication

- Authorization

- Accounting

Benefits

- ✅ Individual user identification

- ✅ User activity tracking

- ✅ Better scalability

- ✅ Centralized credential management

- ✅ Stronger security

📌 Exam association

- Enterprise mode → 802.1X + RADIUS

## 3️⃣ Wireless Security & Encryption Standards

## 🟥 WEP — Wired Equivalent Privacy (DO NOT USE)

- Introduced: 1999

- Original Wi-Fi security standard

- Uses:

  - Pre-shared key

  - RC4 encryption

  - Initialization Vectors (IVs)

Key details

- Original key size: 40-bit

- Later increased to 64-bit and 128-bit

- IV size: 24-bit

- IVs are sent in plain text

Why WEP is broken

- IVs repeat frequently

- Attackers capture enough IVs to:

  - Reverse engineer the key

  - Recover the PSK

- Can be cracked in 2–3 minutes

- Tools like Aircrack-ng easily break it

📌 Exam association

- WEP → Initialization Vector (IV) vulnerability

- Never use WEP

## 🟧 WPA — Wi-Fi Protected Access (OBSOLETE)

- Designed to replace WEP

- Addresses IV weakness

Security features

- Uses TKIP (Temporal Key Integrity Protocol)

  - 48-bit vector (larger than WEP’s IV)

- Still uses RC4 encryption

- Adds MIC (Message Integrity Check)

  - Hashes data to detect modification

  - Helps prevent on-path attacks

Authentication

- Introduced Enterprise mode

- Supports individual credentials via RADIUS

Why WPA is no longer secure

- RC4 encryption is weak

- TKIP is crackable with modern hardware

📌 Exam association

- WPA → TKIP + RC4

## 🟩 WPA2 — Wi-Fi Protected Access 2

- Introduced: 2004

- Based on 802.11i

- Widely used today

Major improvements

- Replaces RC4 with AES

- Replaces MIC with CCMP

Encryption

- AES (Advanced Encryption Standard)

- Key sizes:

  - 128-bit (most common)

  - 192-bit

  - 256-bit

Integrity & confidentiality

- CCMP (Counter Mode with Cipher Block Chaining Message Authentication Code Protocol)

- Provides:

  - Encryption

  - Integrity

  - Authentication

Authentication modes

- Personal mode → PSK

- Enterprise mode → 802.1X + RADIUS

📌 Exam association

- WPA2 → CCMP + AES

## 🟦 WPA3 — Wi-Fi Protected Access 3 (BEST OPTION)

- Introduced: 2018

- Strongest wireless security standard

### WPA3 Personal

- Uses SAE (Simultaneous Authentication of Equals)

- Replaces PSK-based handshake

- Based on Dragonfly key exchange

Why SAE is better

- Resistant to offline dictionary attacks

- Each password guess requires live interaction with the AP

- Brute force attacks become impractical

- Provides forward secrecy

  - If one session key is compromised, past sessions remain secure

### WPA3 Enterprise

- Uses AES-GCM

- Supports stronger encryption:

  - 192-bit

  - 256-bit

📌 Exam association

- WPA3 → SAE + Dragonfly

- Strongest Wi-Fi protection available

## 4️⃣ WPS — Wi-Fi Protected Setup (DISABLE IT)

- Not an encryption protocol

- Designed to simplify Wi-Fi setup

Connection methods

- Push-button

- PIN (8-digit)

- NFC

Why WPS is insecure

- PIN split into two 4-digit halves

- Reduces combinations:

  - From 100,000,000 → ~20,000

- Easily brute-forced in minutes

📌 Exam association

- WPS → Push button + 8-digit PIN

- Should always be disabled

## 5️⃣ Open Wireless Networks

- No authentication

- No encryption

- No password

- Extremely insecure

📌 Exam association

- Open = no WEP, WPA, WPA2, or WPA3

## 6️⃣ Exam Memory Triggers (VERY IMPORTANT)

| Technology | Key Mechanism | Authentication Method | Encryption Used | Critical Detail to Remember | Exam Trigger Thought |
| --- | --- | --- | --- | --- | --- |
| WEP | Initialization Vector (IV) | Pre-Shared Key | RC4 | 24-bit IV sent in plain text; IV reuse allows key recovery | “IV vulnerability” |
| WPA | TKIP | PSK or Enterprise (802.1X) | RC4 | TKIP improves IV handling but still relies on RC4 | “TKIP + RC4” |
| WPA2 | CCMP | PSK or Enterprise (802.1X) | AES | CCMP provides confidentiality and integrity | “AES encryption” |
| WPA3 | SAE (Dragonfly) | SAE (Personal) or Enterprise | AES-GCM | Resistant to offline dictionary attacks; forward secrecy | “SAE handshake” |
| WPS | Push Button / PIN | PIN or Button | Uses WPA/WPA2 keys | 8-digit PIN split into halves → easy brute force | “Disable WPS” |

## 7️⃣ Best Practices (Exam Logic)

- ✅ Use WPA3 whenever possible

- ✅ Use WPA2 if legacy devices exist

- ❌ Never use WEP

- ❌ Never use WPA

- ❌ Disable WPS

# 🌐 Captive Portals

## 1️⃣ What Is a Captive Portal?

A captive portal is a special web page that is automatically displayed to a user after they connect to a network but before they are granted full network access.

- Used on wireless and sometimes wired networks

- The user is “captured” and redirected to the portal

- Access to other network resources is blocked until requirements are met

## 2️⃣ Where Captive Portals Are Commonly Used

Captive portals are heavily used in public and guest networks, including:

- Hotels

- Airports

- Coffee shops

- Restaurants

- Business guest Wi-Fi networks

These environments need controlled access without exposing the internal network.

## 3️⃣ How a Captive Portal Works (Step-by-Step)

- User connects to Wi-Fi or a wired network

- Network intercepts traffic

- User is redirected to the captive portal webpage

- User must complete required actions

- Once approved, the user gains broader network or internet access

## 4️⃣ What a Captive Portal Can Require

A captive portal may require one or more of the following before granting access:

- Accepting terms of service

- Agreeing to network usage policies

- Entering an access code

- Logging in with:

  - Personal credentials

  - Credentials provided by the network owner

- Providing personal information (e.g., email address)

## 5️⃣ Core Functions of Captive Portals

Captive portals provide three key functions:

### A) User Authentication

- Verifies that a user is allowed to access the network

- Helps control who can use the network

### B) Policy Acceptance

- Requires users to agree to:

  - Terms of service

  - Acceptable use policies

- Provides legal protection for the organization

- Ensures users understand the rules before using the network

### C) Data Collection

- Some portals collect user information such as:

  - Email address

  - Name

  - Phone number

- Often used for:

  - Marketing

  - Follow-up communication

  - Promotions

Example from the lesson:

- Coffee shop requires an email address

- Sends a one-time access code by email

- After connection, the user later receives:

  - Promotional emails

  - Discounts

  - Special offers

## 6️⃣ Captive Portals and Guest Networks

- Guest networks are separate from the main internal network

- Designed to:

  - Allow internet access

  - Prevent access to internal systems

- Captive portals are an integral part of guest networks because they:

  - Manage access

  - Enforce rules

  - Control who connects and how

## 7️⃣ Security Benefits of Captive Portals

Captive portals help improve guest network security by:

- Controlling who can access the network

- Tracking network usage

- Reducing risk to the internal network

- Enforcing separation between guest users and internal resources

## 8️⃣ Branding and User Experience

Captive portals can be customized to:

- Display company logos

- Match brand colors and style

- Provide organization-specific information

A well-designed captive portal can:

- Improve user experience

- Provide:

  - Easy connection instructions

  - Troubleshooting tips

  - Information about services or offerings

## 9️⃣ Design & Usability Considerations

A good captive portal should:

- Be easy to navigate

- Provide clear instructions

- Use a straightforward authentication process

- Avoid unnecessary complexity that frustrates users

## 🔐 Data Protection & Legal Compliance

If a captive portal collects personal data, it must:

- Comply with applicable data protection laws

- Protect user privacy

Example regulation mentioned:

- GDPR (General Data Protection Regulation) in the European Union

This is especially important when collecting:

- Names

- Email addresses

- Phone numbers

## 🧪 Testing & Compatibility

Captive portals must be tested to ensure:

- Proper functionality across:

  - Different devices

  - Different operating systems

  - Different browsers

- Users are not blocked due to compatibility issues

## 🔁 Final Summary (Exam-Ready)

- Captive portals are used to control access to public and guest networks

- They balance:

  - User convenience

  - Network security

  - Legal compliance

- They support:

  - Authentication

  - Policy enforcement

  - Data collection

  - Branding

- Proper design, testing, security, and legal compliance are essential

## 🧠 One-Line Exam Definition

A captive portal is a web page that intercepts a user’s network connection and requires authentication or policy acceptance before granting network access.